|  |  |  |  |  |  |  |  |  |  |  |  | Service Report |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Repair Order (RO) Nr. |  |  |  |  | GIS-2, AT0577902 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Date of failure |  |  |  |  | 20.07.2023 ro |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | Repair Order Open Date |  |  |  |  | 21.07.2023 ro |  |  |  |  |  |  |  |
| <<Global Industrial Solutions>> LLC, 100097 Tashkent, Uzbekistan, Tel: +998712319238 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Report to: Rolf Oerter, Distr. Cust. Support Mgr. Tel: +49 1761-5807-737 email: roerter@dgs-mainz.com |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Customer, Orderer: |  |  |  |  |  |  |  |  |  | Enduser, Owner of vehicle: |  |  |  |  |  |  | Location of repair: |  |  |  |  |  |  |
| Name: | Yutong, Allison Transmission |  |  |  |  |  |  |  |  |  | TOSHSHAHARTRANSXIZMAT |  |  |  |  |  |  | Global Industrial Solutions LLC |  |  |  |  |  |  |
| Street: | 1320,Marketing Center, No.66 Yuxing Road |  |  |  |  |  |  |  |  |  | Amir Temur Street, 6 |  |  |  |  |  |  | Yangi Hayot district, Novaya Tashkentskaya Koltsevaya, 120 |  |  |  |  |  |  |
| Town: | Economic Development Zone Zhengzhou, China |  |  |  |  |  |  |  |  |  | Tashkent, Uzbekistan |  |  |  |  |  |  | Tashkent, Uzbekistan |  |  |  |  |  |  |
| ZIP Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Province: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contact: | Yang Wei |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tel.: |  |  |  |  |  |  |  |  |  |  | +998981772552 |  |  |  |  |  |  | +998 71 231 92 38 |  |  |  |  |  |  |
| Fax: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| email: | yangweih@yutong.com |  |  |  |  |  |  |  |  |  | info@tashbus.uz |  |  |  |  |  |  | takhir@global-industrial.uz |  |  |  |  |  |  |
| Customer Purchase order #: |  | Per email of 29.06.2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Customer PO Date: |  |  | 29.06.2023 |  |  |  |
| Repairing Outlet (Allison Dealer) |  |  |  |  |  |  |  |  |  |  | Global Industrial Solutions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transm. Model: |  | T375 |  |  |  |  |  |  |  |  | PN: |  |  |  |  |  |  | SN: |  |  | 6511879170 |  |  |  |
| Replacement Transmission: |  |  |  |  |  |  |  |  |  |  | PN: |  |  |  |  |  |  | SN: |  |  |  |  |  |  |
| TCM, ECU, or Shift Sel. |  |  |  |  |  |  |  |  |  |  | PN 2: |  |  |  |  |  |  | SN 2: |  |  |  |  |  |  |
| Replacement TCM, ECU, or Shift Sel. |  |  |  |  |  |  |  |  |  |  | Replacement PN 2: |  |  |  |  |  |  | Replacement SN 2: |  |  |  |  |  |  |
| Non-Allison Part |  |  |  |  |  |  |  |  |  |  | PN 3: |  |  |  |  |  |  | SN 3: |  |  |  |  |  |  |
| Application: |  | City Bus |  |  |  |  |  |  |  |  | OEM: |  |  | Yutong |  |  |  | Model: |  |  | ZK6126HG |  |  |  |
| Application Code: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vehicle ID, VIN: |  | LZYTMGEF3P1003183 |  |  |  |  |  |  |  |  | Registr.plate: |  |  | Internal number 18335 |  |  |  | Engine: |  | MM5YAP00107 |  |  |  |  |
| In Service Date: |  | 01.04.2023 |  |  |  |  |  |  |  |  | Veh. hours: |  |  |  |  |  |  | Veh.km: |  | 46255 |  |  |  |  |
| Pre Delivery |  | YES |  |  |  | NO |  | X |  |  | Claim Type: |  |  | Machine |  |  |  | Field Action # or Activity Indicator |  |  |  |  |  |  |
| Date in Service after last repair, or Date in Service of exchange unit if failed unit was repaired once before, or if failed unit is already a replacement (SWING) Transmission : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Travel Time: |  |  |  | -- |  |  |  |  | Travel km if company car used: |  |  |  |  |  | -- |  |  | PFPN: |  |  |  |  | 29563171 |  |
| Labor Last Applied date: |  |  |  | 20.08.2023 ro |  |  |  |  | Technician’s Name: |  |  |  |  |  | Gayrat Niyazov |  |  |  |  |  |  |  | AT0577902 |  |
| AWAARE Complaint Code: |  |  | OL06, oil leak |  |  |  | AWAARE Failure Code |  |  |  |  |  | AD04, collapsed |  |  |  |  |  |  |  |  |  |  |  |
| Diagnostic Codes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Complaint: Oil leakage from Retarder Accumulator. Customer could not determinate the source of leakage. Loss of transmission oil.

Cause: Damaged oil seal inside the accumulator.

Correction: R&R Retarder Accumulator. Disassembled / overhauled accumulator and replaced all seals. Added 4 ltr. TES668 fluid.

Comment: Performed an air pressure test to determinate the air pressure in a bus system. Pressure test results – 9,5 bar.  Results were reported to Allison Transmission Europe, Jeroen Nagtzaam.

Failed seals were labelled and were handed to ATE Europe, Jeroen Nagtzaam.

Accumulator test	                  Transmission test

| Labor Code | Description | Time allowed | Time needed |
| --- | --- | --- | --- |
| 00096901 | Troubleshooing and diagnostics | 1,0 | 1,0 |
| 00094011 | R&R underfloor-protection-plate and inner floor plates for access to transmission | 1,0 | 1,0 |
| 00096900 | R&R Retarder Accumulator (0.8), Measuring accumulator apply air pressure (0,4) |  | 1,2 |
| 00095301 | Accumulator Overhaul | 0,3 | 0,3 |
| 00096400 | Final Test drive for leak-test | 0,5 | 0,5 |
|  | Total labor hours |  | 4,0 |

| Part Number | Part Description | Qty |
| --- | --- | --- |
| 29563811 | Seals kit for accumulator was provided by Distr. Tongda Free of Charge | 1 |
|  | See 4WARD claim AT0568460 |  |

| Net item type | Net item Description | Gross (incl. VAT) | VAT | Net |
| --- | --- | --- | --- | --- |
| -M- | 4 Liters TES668 fluid |  |  |  |
| -X- | 12% Uzbekistan VAT on total request of claim |  |  | 25,35 € |
| -F- |  |  |  |  |